## N.E.T. 2026 – 3D Printing Task Sheet

Theme: Engineering Solutions for Sustainable Cities

Time Allotted: 120 Minutes (14:00 – 16:00)

### 1. The Design Challenge

You are required to optimize and prepare for printing a structural bracket or a modular connector (the base STL file is provided on your workstation). The object must be able to support a specific load while minimizing the material used (weight).

### 2. Technical Requirements (Slicing Process)

Using the provided slicing software, configure the following parameters:

- Layer Height: Between 0.15mm and 0.25mm for a balance between speed and strength.

- Infill Pattern: Select a pattern that offers the best structural integrity (e.g., Gyroid or Honeycomb).

- Support Material: Enable supports only where necessary (overhangs > 45°).

- Wall Count: Minimum 3 perimeters to ensure durability.

### 3. Practical Printing Execution

- Bed Adhesion: Ensure the print surface is clean and use appropriate adhesion settings (Brim/Raft) if the geometry requires it.

- Monitoring: You must monitor the first 3 layers to prevent warping or detachment.

- Post-processing: Once the print is finished (or a designated sample section), carefully remove supports without damaging the main structure.

### 4. Written Technical Justification (English)

Provide a brief technical report (max. 100 words) addressing the following points in English:

- Why did you choose this specific Infill Density?

- How did you optimize the Print Orientation to reduce support waste?

- Describe one challenge you encountered during the slicing or printing phase and how you solved it.

### Evaluation Checklist for Supervisors:

- File Readiness: Is the G-code optimized for the specific printer model?

- Resource Management: Did the student minimize filament waste?

- Safety Protocols: Did the student follow the laboratory safety rules during printer operation?